# HC-77B Rev A

## Technical Specification

### Final draft baseline

# 1. Product definition

Model: HC-77B Rev A
Launch year: 1977
Type: assembled single-board microcomputer
Positioning: serious expandable base computer, above a pure trainer, below a full keyboard/video home computer

## Design goals

- feel like a real computer, not a toy

- remain price-disciplined

- support a genuine early software ecosystem

- allow practical expansion without over-engineering Rev A

- create a believable springboard to the 1978/79 bundle phase and 1980 relaunch

# 2. Core hardware

## CPU

- MOS Technology 6502

- Clock: 1 MHz nominal

## Memory

- RAM standard: 1 KB

- Official RAM expansion ceiling: 4 KB

- ROM: 2 KB monitor ROM

## Base user interface

- Display: 8-digit seven-segment LED display

- Input: hexadecimal keypad with dedicated monitor keys

## Standard I/O/features

- cassette interface fitted as standard

- serial TTY interface supported

- optional ASCII keyboard interface

- minimal onboard monochrome text video generation

- external TV modulator sold separately

# 3. Physical form

## Board format

- assembled bare-board product

- no full case at launch

- designed for desk/bench use

- supplied with PSU and printed manual

## Sold separately

- cassette leads

- TV modulator

- RAM expansion

- Tiny BASIC cassette

- later ROM BASIC option

- keyboard/interface add-ons

- display/TV accessories

# 4. Functional personality

HC-77B is a front-panel computer first.

It is intended to be usable in three progressively richer ways:

## Level 1: front-panel only

- memory entry

- memory inspection

- basic execution

- monitor usage

- tape save/load

## Level 2: front-panel + cassette

- practical program storage

- Tiny BASIC loading

- software distribution

- user program retention

## Level 3: expanded system

- video output via modulator

- ASCII keyboard

- RAM expansion

- fuller BASIC

- more comfortable personal-computing use

# 5. Memory architecture

## Philosophy

- conventional 6502-style low RAM / top ROM arrangement

- simple to understand

- simple to emulate

- enough reserved space for future growth

- not over-architected

## Draft memory map

$0000-$03FF   1 KB RAM

$0400-$07FF   reserved / future RAM expansion

$0800-$08FF   I/O page

$0900-$0FFF   reserved peripheral / video / interface space

$1000-$F7FF   expansion address space

$F800-$FFFF   2 KB monitor ROM

## Notes

- zero page and stack live within base RAM in the normal 6502 manner

- reset/NMI/IRQ vectors are provided in top ROM

- RAM expansion to 4 KB is the official supported limit for Rev A lineage

# 6. I/O architecture

## Philosophy

Use:

- simple direct logic where the machine should feel tactile and immediate

- slightly smarter interface logic where timing and protocol matter

## Chosen approach

Restrained PIA-like smart I/O, used sparingly.

This means:

- front-panel display and keypad remain conceptually simple

- cassette and serial support gain limited smarter control

- Rev A remains authentic and not over-integrated

## Functional I/O regions

Suggested draft mapping:

$0800  keypad row select / control

$0801  keypad read / monitor key state

$0802  LED digit select

$0803  LED segment data

$0804  cassette data/control

$0805  cassette status / verify sense

$0806  serial/TTY data

$0807  serial/TTY status/control

$0808-$08FF reserved

## Important note

These addresses are now the working canonical map unless we later find a compelling reason to reshuffle them.

# 7. Display system

## Front-panel display

- 8-digit LED

- seven-segment numeric/hex display

- sufficient for:

  - address/data work

  - mode feedback

  - status presentation

## Power-on behavior

On reset:

- brief LED segment test

- short identifier pattern, e.g. HC77

- transition to normal monitor display state

## Why 8 digits

Chosen over 6 digits because:

- more serious product feel

- better usability

- more credible as a base system

- less trainer-like

- still affordable at the intended price level

# 8. Keypad layout

## Entry keys

- 0–9

- A–F

## Dedicated monitor keys

Canonical Rev A key set:

- ADR

- DAT

- INC

- DEC

- GO

- REG

- LOAD

- SAVE

- RST

## Interaction style

Front-panel first:

- choose address mode

- enter hex address

- choose data mode

- enter/inspect bytes

- step through memory

- invoke run or cassette operations directly

This is intentionally not a text-command monitor.

# 9. Monitor ROM

## ROM size

- 2 KB

## Monitor style

A-plus monitor

- minimal in spirit

- but includes what is necessary for cassette to be a core system feature

## Included functions

- cold reset / initialization

- front-panel ready state

- memory examine

- memory modify

- address stepping

- run/go to address

- register view/edit

- cassette save

- cassette load

- basic verify / error indication

## Excluded from Rev A

- mini-assembler

- disassembler

- advanced debugger

- rich trace system

- complex command parser

## Rationale

The ROM should support the machine’s identity without undermining:

- Tiny BASIC on cassette

- later fuller BASIC

- later ROM add-ons

- the value of RAM expansion

# 10. Video system

## Built-in video philosophy

Minimal video logic is present on the base board.

This is deliberate:

- more serious than a pure trainer

- less complete than a full board-computer

- supports an upgrade path without feeling too modular

## Video output

- monochrome text generation on board

- TV modulator required for practical TV connection

- modulator sold separately

## Text mode

- 16 rows × 32 columns

## Rev A limits

- text only

- no proper bitmap graphics mode

- no claim of a full-screen home-computer experience

- practical use strongly improved by keyboard and RAM expansion

# 11. Cassette system

## Cassette status

- standard on every HC-77B

This is a core product decision:

- backing store is essential

- tape support differentiates the machine from toy-like trainer boards

- company software sales depend on it

## Physical encoding

- Kansas City Standard–style

- 300 baud

- 1200/2400 Hz FSK logic

## Logical format

Use a simple HC block format over KCS-style physical encoding.

### Proposed block structure

- leader tone

- sync byte/pattern

- start address

- length

- data bytes

- checksum

- trailer/end

## Monitor support

- LOAD

- SAVE

- basic verify/error reporting

## Later evolution

- later systems may introduce higher-speed or enhanced formats

- Rev A remains compatible in principle with the early company tape lineage

# 12. Keyboard and terminal strategy

## Standard

- no full keyboard included at launch

- front panel remains the primary interface

## Supported from Rev A

- serial TTY connection

- optional ASCII keyboard interface path

## Philosophy

This gives the machine seriousness and room to grow without making Rev A a half-finished keyboard computer.

# 13. RAM and software strategy

## Base RAM

- 1 KB standard

## Expansion

- 4 KB official supported maximum for HC-77B class

## Software implications

This keeps Rev A disciplined and ensures the larger leap belongs to the 1980 machines.

## BASIC plan

### Shortly after launch

- Tiny BASIC on cassette

### Around 1978/79 bundle phase

- fuller BASIC in ROM

This staged software path is a key part of the commercial ladder.

# 14. Commercial configuration

## Launch sales form

- assembled only

## Why

- more serious company image

- broader buyer appeal than kit-only

- fewer field failures

- cleaner support story

- stronger software market consistency

## Launch package contents

- assembled HC-77B board

- PSU

- printed manual

## Not included

- cassette leads

- recorder

- TV modulator

- keyboard

- RAM expansion

- BASIC

# 15. Product positioning summary

## HC-77B is not:

- a cheapest-possible trainer

- a hobbyist parts kit

- a full keyboard/video personal computer

- a clone of KIM-1, MK14, Nascom, or Superboard

## HC-77B is:

- a serious assembled 1977 single-board computer

- front-panel first

- tape-capable from day one

- minimally video-capable

- upgrade-led

- commercially disciplined

- the seed of a broader product family

16. Canonical working spec sheet

| Category | Final choice |
| --- | --- |
| CPU | MOS 6502 @ 1 MHz |
| RAM | 1 KB standard |
| RAM expansion | 4 KB max official |
| ROM | 2 KB monitor ROM |
| Display | 8-digit LED |
| Input | hex keypad + dedicated monitor keys |
| Monitor | front-panel-first, A-plus |
| Cassette | standard |
| Tape encoding | KCS-style 300 baud |
| Tape format | HC structured block format |
| Serial | TTY supported |
| Keyboard | optional ASCII keyboard path |
| Video | minimal built-in monochrome text logic |
| Text mode | 16 × 32 |
| TV output | modulator sold separately |
| I/O style | restrained PIA-like smart I/O |
| Launch form | assembled only |
| In box | board + PSU + manual |
| Early software | Tiny BASIC on cassette |
| Later software | fuller BASIC in ROM around 78/79 |
| Power-on | segment test → HC77 ID → monitor ready |

# 17. What remains to define later

This technical spec is now strong enough to anchor the project, but a few lower-level details still need freezing when we move into emulator/spec-implementation mode:

- exact LED segment encoding

- keypad matrix wiring

- exact register display format

- exact cassette sync/checksum bytes

- exact serial framing conventions

- exact monitor ROM state-machine flow

- exact reset vector behavior

- exact optional keyboard electrical interface

Those are now implementation details, not “what kind of machine is this?” questions.